Computer Networks COE 351

# Final Project

Contents

Final Project	1

1.	IP's for the routers	3

why we used that?	4

2.	New areas ip addresses	5

Area 47	5

Area 48	5

Area 49	6

Area 50	6

3.	For the LAN's:	6

3.1	LAN in area 1 should have around 60000 users.	7

3.2	Area 2 should contain two VLANs:	7

3.3	LAN in area 3 should have around 8000 users.	8

3.4	Lan in area 4 should have 500 users.	9

Address table after add all interfaces	10

OSPF routing protocol	12

R12 Show ospf neighbor	14

R17 Show ospf neighbor	14

R14 Show ospf neighbor	15

R12 Show ip ospf database	15

R14 Show ip ospf database	16

R17 Show ip ospf database	17

R14 Show ip ospf database summary	18

R17 Show ip ospf database summary	20

R12 Show ip ospf database summary	24

cost from Router 14 to Router 17	27

show ip ospf interface brief	29

Shortest path from Router 14 to Router 17	31

Cost from Router 14 to Router 17	31

Router 11	37

Router 12	40

Router 2	42

Router 13	45

Router 4	47

Router 17	49

Router 16	52

Router 15	54

Router 14	56

Figure 1 Topology	4

Figure 2 Topology with added addresses	5

Figure 3 R12 Show ospf neighbor	16

Figure 4 R17 Show ospf neighbor	16

Figure 5 R14 Show ospf neighbor	17

Figure 6 R12 Show ip ospf database	18

Figure 7 R14 Show ip ospf database	19

Figure 8 R17 Show ip ospf database	20

Figure 9 R14 Show ip ospf database summary	21

Figure 10  R14 Show ip ospf database summary	22

Figure 11 R17 Show ip ospf database summary	22

Figure 12 R17 Show ip ospf database summary	24

Figure 13  R17 Show ip ospf database summary	25

Figure 14 R12 Show ip ospf database summary	26

Figure 15 R12 Show ip ospf database summary	27

Figure 16 R12 Show ip ospf database summary	28

Figure 17 topology	30

Figure 18 optimal path from R14 to R17	30

Figure 19 Router 14 show ip ospf interface brief	31

Figure 20 Router 12 show ip ospf interface brief	31

Figure 21 Router 2 show ip ospf interface brief	31

Figure 22 Router 17 show ip ospf interface brief	31

Figure 23 Router 11  show ip ospf interface brief	32

Figure 24 Router 13 show ip ospf interface brief	32

Figure 25 cost at each path	32

Figure 26 Router 12 show ip route ospf	33

Figure 27 shortest path cost selection	34

Figure 28 Router 2 show ip ospf	35

Figure 29 Router 12 show ip ospf	36

Figure 30 Router 14 show ip ospf	37

Figure 31 Router 11 show ip ospf	38

Figure 32 Router 13  show ip ospf	39

Table 1 addresssing table	5

Table 2 area table	6

Table 3 address range in each area	7

Table 4 area 47	7

Table 5 area 48	7

Table 6 area 49	8

Table 7 area 50	8

Table 8 lan in area 47	9

Table 9 lan in area 48	10

Table 10 lan in area 49	11

Table 11 lan in area 50	11

Table 12 whole lans in each area	12

Table 13 whole addressing table	13

Table 14 ospf configuration in each router	14

Table 15 area names and addresses	29

Figure 1 Topology

You need to do the followed:

- Choose the IP's for the routers and use subnet 30, and write why we used that? (write the IP addresses next to the interface).

# IP's for the routers

Figure 2 Topology with added addresses

Student id : 391202847

x = 47

Table 1 addresssing table

| Router 11 |  |  |
| --- | --- | --- |
| Fa5/0 | 47.0.0.1 | 255.255.255.252 |
| Fa0/0 | 47.0.0.5 | 255.255.255.252 |
| F4/0 | 47.0.0.9 | 255.255.255.252 |
| Router 12 |  |  |
| Fa5/0 | 47.0.0.10 | 255.255.255.252 |
| Fa4/0 | 47.0.0.21 | 255.255.255.252 |
| Fa0/0 | 47.0.0.17 | 255.255.255.252 |
| Router 2 |  |  |
| Fa5/0 | 47.0.0.2 | 255.255.255.252 |
| Fa0/0 | 47.0.0.18 | 255.255.255.252 |
| Fa4/0 | 47.0.0.13 | 255.255.255.252 |
| Fa1/0 | 47.0.0.26 | 255.255.255.252 |
| Router 13 |  |  |
| Fa4/0 | 47.0.0.14 | 255.255.255.252 |
| Fa0/0 | 47.0.0.6 | 255.255.255.252 |
| Fa5/0 | 47.0.0.22 | 255.255.255.252 |

# why we used that?

Table 2 area table

| 47.0.0.0 /30 | 47.0.0.1 | 47.0.0.2 | 255.255.255.252 |
| --- | --- | --- | --- |
| 47.0.0.4/30 | 47.0.0.5 | 47.0.0.6 | 255.255.255.252 |
| 47.0.0.8/30 | 47.0.0.9 | 47.0.0.10 | 255.255.255.252 |
| 47.0.0.12/30 | 47.0.0.13 | 47.0.0.14 | 255.255.255.252 |
| 47.0.0.16/30 | 47.0.0.17 | 47.0.0.18 | 255.255.255.252 |
| 47.0.0.20/30 | 47.0.0.21 | 47.0.0.22 | 255.255.255.252 |

- Area number should match of X except the one in area 0. EX if IP is 10.10.0.0 area should be area 10. For LANs use 192.168.X.0 change X to be the same as your area ID. on area 2, the second VLAN X will be (area ID + 4).

# New areas ip addresses

Table 3 address range in each area

| Area 0 |  |  |
| --- | --- | --- |
| Area 1 = > Area 47 |  |  |
| Area Subnet | Start IP | second IP |
| 47.0.0.24/30 | 47.0.0.25 | 47.0.0.26 |
| Area 2 = > Area 48 |  |  |
| Area Subnet | Start IP | second IP |
| 48.0.0.24/30 | 48.0.0.25 | 48.0.0.26 |
| Area 3 = > Area 49 |  |  |
| Area Subnet | Start IP | second IP |
| 49.0.0.24/30 | 49.0.0.25 | 49.0.0.26 |
| 49.0.0.28/30 | 49.0.0.29 | 49.0.0.30 |
| Area 4 = > Area 50 |  |  |
| Area Subnet | Start IP | second IP |
| 50.0.0.24/30 | 50.0.0.25 | 50.0.0.26 |

# Area 47

Table 4 area 47

| Area 47 |  |  |
| --- | --- | --- |
| Router 2 |  |  |
| Fa1/0 | 47.0.0.26 | 255.255.255.252 |
| Router 17 |  |  |
| Fa0/0 | 47.0.0.25 | 255.255.255.252 |

# Area 48

Table 5 area 48

| Area 48 |  |  |
| --- | --- | --- |
| Router 13 |  |  |
| Fa1/0 | 48.0.0.25 | 255.255.255.252 |
| Router 16 |  |  |
| Fa0/0 | 48.0.0.26 | 255.255.255.252 |

# Area 49

Table 6 area 49

| Area 49 |  |  |
| --- | --- | --- |
| Router 14 |  |  |
| Fa0/0 | 49.0.0.26 | 255.255.255.252 |
| Router 15 |  |  |
| Fa0/0 | 49.0.0.29 | 255.255.255.252 |
| Router 12 |  |  |
| Fa1/0 | 49.0.0.25 | 255.255.255.252 |
| Fa7/0 | 49.0.0.30 | 255.255.255.252 |

# Area 50

Table 7 area 50

| Area 50 |  |  |
| --- | --- | --- |
| Router 11 |  |  |
| Fa1/0 | 50.0.0.25 | 255.255.255.252 |
| Router 4 | Area 50 |  |
| Fa0/0 | 50.0.0.26 | 255.255.255.252 |

# For the LAN's:

1. LAN in area 3 should have around 8000 users. (chose the closest number to 8000 users but not less than 8000 users).

2. Lan in area 4 should have 500 users.

3. Area 2 should contain two VLANs:

1. VLAN 1 should be class C

2. VLAN 2 should have 3000 users.

4. LAN in area 1 should have around 60000 users.

4. Each LAN and VLAN should have DHCP service.

For LAN in each area

# LAN in area 1 should have around 60000 users.

LAN : 192.168.47.0/16 = 65536 users

Enable DHCP service

Start	192.168.0.1	End	192.168.255.254

Table 8 lan in area 47

Area 47

Router 17

! ip dhcp pool router17 network 192.168.74.0 255.255.0.0 default-router 192.168.0.1

# Area 2 should contain two VLANs:

1. VLAN 1 should be class C

2. VLAN 2 should have 3000 users.

LAN 192.168.66.0/ 24= class C

Enable DHCP service

Start	192.168.66.1	End	192.168.66.254

LAN 192.168.56.0/ 20= 4094 users

Enable DHCP service

Start	192.168.16.1	End	192.168.31.254

Table 9 lan in area 48

Area 48

Router 16

! ip dhcp pool router16 network 192.168.66.0 255.255.255.0 default-router 192.168.66.1 ! ip dhcp pool router16-2 network 192.168.56.0 255.255.240.0 default-router 192.168.16.1

# LAN in area 3 should have around 8000 users.

(chose the closest number to 8000 users but not less than 8000 users).

LAN  192.168.31.0/24  class C

Enable DHCP service

Start	192.168.31.1	end	192.168.31.254

LAN  192.168.49.0/19 = 8,192 users

Enable DHCP service

Start	192.168.32.1	end	192.168.63.254

Table 10 lan in area 49

Area 49

Router 14

! ip dhcp pool router14 network 192.168.49.0 255.255.224.0 default-router 192.168.32.1 ! ! ip dhcp pool router14-2 network 192.168.31.0 255.255.255.0 default-router 192.168.31.1 !

# Lan in area 4 should have 500 users.

LAN : 192.168.64.0/23= 512 users

Enable DHCP service

End	192.168.65.254

Table 11 lan in area 50

Area 50

Router 4

! ip dhcp pool router4 network 192.168.49.0 255.255.254.0 default-router 192.168.64.1

Table 12 whole lans in each area

| Area 47 |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| LAN : 192.168.47.0/16 = 65536 users |  |  |  |  |  |  |  |  |  |  |  |  |
| Start |  |  |  | 192.168.0.1 |  |  |  | End |  |  |  | 192.168.255.254 |
| Area 48 |  |  |  |  |  |  |  |  |  |  |  |  |
| LAN 192.168.66.0/ 24= class C |  |  |  |  |  |  |  |  |  |  |  |  |
| LAN 192.168.56.0/ 20= 4094 users |  |  |  |  |  |  |  |  |  |  |  |  |
| Start |  |  | 192.168.16.1 |  |  |  | End |  |  |  | 192.168.31.254 |  |
| Area 49 |  |  |  |  |  |  |  |  |  |  |  |  |
| LAN 192.168.31.0/24 class C |  |  |  |  |  |  |  |  |  |  |  |  |
| LAN 192.168.49.0/19 = 8,192 users |  |  |  |  |  |  |  |  |  |  |  |  |
| Start | 192.168.32.1 |  |  |  | end |  |  |  | 192.168.63.254 |  |  |  |
| Area 50 |  |  |  |  |  |  |  |  |  |  |  |  |
| LAN : 192.168.64.0/23= 512 users |  |  |  |  |  |  |  |  |  |  |  |  |
| Start |  | 192.168.64.1 |  |  |  | End |  |  |  | 192.168.65.254 |  |  |

# Address table after add all interfaces

Table 13 whole addressing table

| Router 11 |  |  |
| --- | --- | --- |
| Fa5/0 | 47.0.0.1 | 255.255.255.252 |
| Fa0/0 | 47.0.0.5 | 255.255.255.252 |
| F4/0 | 47.0.0.9 | 255.255.255.252 |
| Fa1/0 | 50.0.0.25 | 255.255.255.252 |
| Router 12 |  |  |
| Fa5/0 | 47.0.0.10 | 255.255.255.252 |
| Fa4/0 | 47.0.0.21 | 255.255.255.252 |
| Fa0/0 | 47.0.0.17 | 255.255.255.252 |
| Fa1/0 | 49.0.0.25 | 255.255.255.252 |
| Fa7/0 | 49.0.0.30 | 255.255.255.252 |
| Router 2 |  |  |
| Fa5/0 | 47.0.0.2 | 255.255.255.252 |
| Fa0/0 | 47.0.0.18 | 255.255.255.252 |
| Fa4/0 | 47.0.0.13 | 255.255.255.252 |
| Fa1/0 | 47.0.0.26 | 255.255.255.252 |
| Router 13 |  |  |
| Fa4/0 | 47.0.0.14 | 255.255.255.252 |
| Fa0/0 | 47.0.0.6 | 255.255.255.252 |
| Fa5/0 | 47.0.0.22 | 255.255.255.252 |
| Fa1/0 | 48.0.0.25 | 255.255.255.252 |
| Router 4 | Area 50 |  |
| Fa0/0 | 50.0.0.26 | 255.255.255.252 |
| LO4 | 192.168.13.0 | 255.255.255.0 |
| LAN | 192.168.64.0 | 255.255.254.0 |
| Router 14 | Area 49 |  |
| Fa0/0 | 49.0.0.26 | 255.255.255.252 |
| LO3 | 192.168.12.0 | 255.255.255.0 |
| Router 15 | Area 49 |  |
| Fa0/0 | 49.0.0.29 | 255.255.255.252 |
| LAN | 192.168.49.0 | 255.255.224.0 |
| Router 17 | Area 47 |  |
| Fa0/0 | 47.0.0.25 | 255.255.255.252 |
| LO1 | 192.168.10.0 | 255.255.255.0 |
| LAN | 192.168.47.0 | 255.255.0.0 |
| Router 16 | Area 48 |  |
| Fa0/0 | 48.0.0.26 | 255.255.255.252 |
| LO2 | 192.168.11.0 | 255.255.255.0 |
| LAN | 192.168.66.0 | 255.255.255.0 |
| LAN | 192.168.56.0 | 255.255.240.0 |

# OSPF routing protocol

- Use OSPF routing protocol with the areas shown on the figure. (copy the commands on your report)

Table 14 ospf configuration in each router

Router 11 router ospf 1 router-id 11.11.11.11 log-adjacency-changes network 47.0.0.0 0.0.0.3 area 0 network 47.0.0.4 0.0.0.3 area 0 network 47.0.0.8 0.0.0.3 area 0 network 50.0.0.24 0.0.0.3 area 50 !

Router 2 ! router ospf 1 router-id 2.2.2.2 log-adjacency-changes network 47.0.0.0 0.0.0.3 area 0 network 47.0.0.12 0.0.0.3 area 0 network 47.0.0.16 0.0.0.3 area 0 network 47.0.0.24 0.0.0.3 area 47 !

Router 12 ! router ospf 1 router-id 12.12.12.12 log-adjacency-changes network 47.0.0.8 0.0.0.3 area 0 network 47.0.0.16 0.0.0.3 area 0 network 47.0.0.20 0.0.0.3 area 0 network 49.0.0.24 0.0.0.3 area 49 network 49.0.0.28 0.0.0.3 area 49 !

Router 13 ! router ospf 1 router-id 13.13.13.13 log-adjacency-changes network 47.0.0.12 0.0.0.3 area 0 network 47.0.0.4 0.0.0.3 area 0 network 47.0.0.20 0.0.0.3 area 0 network 48.0.0.24 0.0.0.3 area 48 !

Router 4 ! router ospf 1 log-adjacency-changes network 50.0.0.24 0.0.0.3 area 50

Router 17 router ospf 1 log-adjacency-changes network 47.0.0.24 0.0.0.3 area 47 !

Router 16 ! router ospf 1 log-adjacency-changes network 48.0.0.24 0.0.0.3 area 48 !

Router 14 ! router ospf 1 log-adjacency-changes network 49.0.0.24 0.0.0.3 area 49 !

Router 15 ! router ospf 1 log-adjacency-changes network 49.0.0.28 0.0.0.3 area 49 !

6. Use show command to show the next for router 12, router 17, router 14:

- neighbors

# R12 Show ospf neighbor

Figure 3 R12 Show ospf neighbor

# R17 Show ospf neighbor

Figure 4 R17 Show ospf neighbor

# R14 Show ospf neighbor

Figure 5 R14 Show ospf neighbor

- database

# R12 Show ip ospf database

Figure 6 R12 Show ip ospf database

# R14 Show ip ospf database

Figure 7 R14 Show ip ospf database

# R17 Show ip ospf database

Figure 8 R17 Show ip ospf database

- database summary.

# R14 Show ip ospf database summary

Figure 9 R14 Show ip ospf database summary

Figure 10  R14 Show ip ospf database summary

# R17 Show ip ospf database summary

Figure 11 R17 Show ip ospf database summary

Figure 12 R17 Show ip ospf database summary

Figure 13  R17 Show ip ospf database summary

# R12 Show ip ospf database summary

Figure 14 R12 Show ip ospf database summary

Figure 15 R12 Show ip ospf database summary

Figure 16 R12 Show ip ospf database summary

7. Chose the area type carefully and write the reason why have chosen it for each area.

Table 15 area names and addresses

| Area 0 |  |  |
| --- | --- | --- |
| Area 1 = Area 47 |  |  |
| 47.0.0.24/30 | 47.0.0.25 | 47.0.0.26 |
| Area 2 = Area 47+1 = Area 48 |  |  |
| 48.0.0.24/30 | 48.0.0.25 | 48.0.0.26 |
| Area 3 = Area x+2 = 47+2 = 49 |  |  |
| 49.0.0.24/30 | 49.0.0.25 | 49.0.0.26 |
| 49.0.0.28/30 | 49.0.0.29 | 49.0.0.30 |
| Area 4 = Area x+3 = 50 |  |  |
| 50.0.0.24/30 | 50.0.0.25 | 50.0.0.26 |

# cost from Router 14 to Router 17

8. Show the cost from Router 14 to Router 17 and show all the paths. Define which one is shorter. Do Q.8 with the default reference B/W and see if you have to change it, Explain.

Figure 17 topology

Figure 18 optimal path from R14 to R17

# show ip ospf interface brief

Figure 19 Router 14 show ip ospf interface brief

Figure 20 Router 12 show ip ospf interface brief

Figure 21 Router 2 show ip ospf interface brief

Figure 22 Router 17 show ip ospf interface brief

Figure 23 Router 11  show ip ospf interface brief

Figure 24 Router 13 show ip ospf interface brief

Figure 25 cost at each path

# Shortest path from Router 14 to Router 17

R14 > R12 > R2 > R17 is the shortest path

Figure 26 Router 12 show ip route ospf

# Cost from Router 14 to Router 17

8. Show the cost from Router 14 to Router 17 with the default reference B/W and see if you have to change it, Explain.

All interfaces are fastEthernet which are equal all over this topology

Figure 27 shortest path cost selection

R14 > R12 > R2 > R17 is the shortest path

This path remains the optimal path based on b/w

Figure 28 Router 2 show ip ospf

Figure 29 Router 12 show ip ospf

Figure 30 Router 14 show ip ospf

Figure 31 Router 11 show ip ospf

Figure 32 Router 13  show ip ospf

# Router 11

Router11#show running-config Building configuration... Current configuration : 1048 bytes ! version 12.2 no service timestamps log datetime msec no service timestamps debug datetime msec no service password-encryption ! hostname Router11 ! ! ! ! ! ! ! ! no ip cef no ipv6 cef ! ! ! ! ! ! ! ! ! ! ! ! ! ! ! ! ! ! interface FastEthernet0/0 ip address 47.0.0.1 255.255.255.252 duplex auto speed auto ! interface FastEthernet1/0 ip address 50.0.0.25 255.255.255.252 duplex auto speed auto ! interface Serial2/0 no ip address clock rate 2000000 shutdown ! interface Serial3/0 no ip address clock rate 2000000 shutdown ! interface FastEthernet4/0 ip address 47.0.0.9 255.255.255.252 ! interface FastEthernet5/0 ip address 47.0.0.5 255.255.255.252 ! interface Modem8/0 no ip address ! interface Modem9/0 no ip address ! router ospf 1 router-id 11.11.11.11 log-adjacency-changes network 47.0.0.0 0.0.0.3 area 0 network 47.0.0.4 0.0.0.3 area 0 network 47.0.0.8 0.0.0.3 area 0 network 50.0.0.24 0.0.0.3 area 50 ! ip classless ! ip flow-export version 9 ! ! ! ! ! ! ! ! line con 0 ! line aux 0 ! line vty 0 4 login ! ! ! end Router11#

# Router 12

Router12#show running-config Building configuration... Current configuration : 1263 bytes ! version 12.2 no service timestamps log datetime msec no service timestamps debug datetime msec no service password-encryption ! hostname Router12 ! ! ! ! ! ! ! ! no ip cef no ipv6 cef ! ! ! ! ! ! ! ! ! ! ! ! ! ! ! ! ! ! interface FastEthernet0/0 ip address 47.0.0.17 255.255.255.252 duplex auto speed auto ! interface FastEthernet1/0 ip address 49.0.0.30 255.255.255.252 duplex auto speed auto ! interface Serial2/0 no ip address clock rate 2000000 shutdown ! interface Serial3/0 no ip address clock rate 2000000 shutdown ! interface FastEthernet4/0 ip address 47.0.0.21 255.255.255.252 ! interface FastEthernet5/0 ip address 47.0.0.10 255.255.255.252 ! interface FastEthernet6/0 no ip address duplex auto speed auto shutdown ! interface FastEthernet7/0 ip address 49.0.0.25 255.255.255.252 duplex auto speed auto ! interface FastEthernet9/0 no ip address duplex auto speed auto shutdown ! router ospf 1 router-id 12.12.12.12 log-adjacency-changes network 47.0.0.8 0.0.0.3 area 0 network 47.0.0.16 0.0.0.3 area 0 network 47.0.0.20 0.0.0.3 area 0 network 49.0.0.24 0.0.0.3 area 49 network 49.0.0.28 0.0.0.3 area 49 ! ip classless ! ip flow-export version 9 ! ! ! ! ! ! ! ! line con 0 ! line aux 0 ! line vty 0 4 login ! ! ! end Router12#

# Router 2

Router#show running-config Building configuration... Current configuration : 1122 bytes ! version 12.2 no service timestamps log datetime msec no service timestamps debug datetime msec no service password-encryption ! hostname Router ! ! ! ! ! ! ! ! no ip cef no ipv6 cef ! ! ! ! ! ! ! ! ! ! ! ! ! ! ! ! ! ! interface FastEthernet0/0 ip address 47.0.0.18 255.255.255.252 duplex auto speed auto ! interface FastEthernet1/0 ip address 47.0.0.26 255.255.255.252 duplex auto speed auto ! interface Serial2/0 no ip address clock rate 2000000 shutdown ! interface Serial3/0 no ip address clock rate 2000000 shutdown ! interface FastEthernet4/0 ip address 47.0.0.13 255.255.255.252 ! interface FastEthernet5/0 ip address 47.0.0.2 255.255.255.252 ! interface Ethernet8/0 no ip address duplex auto speed auto shutdown ! interface Ethernet9/0 no ip address duplex auto speed auto shutdown ! router ospf 1 router-id 2.2.2.2 log-adjacency-changes network 47.0.0.0 0.0.0.3 area 0 network 47.0.0.12 0.0.0.3 area 0 network 47.0.0.16 0.0.0.3 area 0 network 47.0.0.24 0.0.0.3 area 47 ! ip classless ! ip flow-export version 9 ! ! ! ! ! ! ! ! line con 0 ! line aux 0 ! line vty 0 4 login ! ! ! end Router#

# Router 13

Router13#show running-config Building configuration... Current configuration : 1128 bytes ! version 12.2 no service timestamps log datetime msec no service timestamps debug datetime msec no service password-encryption ! hostname Router13 ! ! ! ! ! ! ! ! no ip cef no ipv6 cef ! ! ! ! ! ! ! ! ! ! ! ! ! ! ! ! ! ! interface FastEthernet0/0 ip address 47.0.0.6 255.255.255.252 duplex auto speed auto ! interface FastEthernet1/0 ip address 48.0.0.25 255.255.255.252 duplex auto speed auto ! interface Serial2/0 no ip address clock rate 2000000 shutdown ! interface Serial3/0 no ip address clock rate 2000000 shutdown ! interface FastEthernet4/0 ip address 47.0.0.14 255.255.255.252 ! interface FastEthernet5/0 ip address 47.0.0.22 255.255.255.252 ! interface Ethernet8/0 no ip address duplex auto speed auto shutdown ! interface Ethernet9/0 no ip address duplex auto speed auto shutdown ! router ospf 1 router-id 13.13.13.13 log-adjacency-changes network 47.0.0.12 0.0.0.3 area 0 network 47.0.0.4 0.0.0.3 area 0 network 47.0.0.20 0.0.0.3 area 0 network 48.0.0.24 0.0.0.3 area 48 ! ip classless ! ip flow-export version 9 ! ! ! ! ! ! ! ! line con 0 ! line aux 0 ! line vty 0 4 login ! ! ! end Router13#

# Router 4

Router#sh run Building configuration... Current configuration : 960 bytes ! version 12.2 no service timestamps log datetime msec no service timestamps debug datetime msec no service password-encryption ! hostname Router ! ! ! ! ! ip dhcp pool router4 network 192.168.48.0 255.255.254.0 default-router 192.168.64.1 ! ! ! ip cef no ipv6 cef ! ! ! ! ! ! ! ! ! ! ! ! ! ! ! ! ! ! interface Loopback4 ip address 192.168.13.1 255.255.255.0 ! interface FastEthernet0/0 ip address 50.0.0.26 255.255.255.252 duplex auto speed auto ! interface FastEthernet1/0 no ip address duplex auto speed auto shutdown ! interface Serial2/0 no ip address clock rate 2000000 shutdown ! interface Serial3/0 no ip address clock rate 2000000 shutdown ! interface FastEthernet4/0 no ip address shutdown ! interface FastEthernet5/0 no ip address shutdown ! router ospf 1 log-adjacency-changes network 50.0.0.24 0.0.0.3 area 50 ! ip classless ! ip flow-export version 9 ! ! ! ! ! ! ! ! line con 0 ! line aux 0 ! line vty 0 4 login ! ! ! end Router#

# Router 17

Router17#sh run Building configuration... Current configuration : 959 bytes ! version 12.2 no service timestamps log datetime msec no service timestamps debug datetime msec no service password-encryption ! hostname Router17 ! ! ! ! ! ip dhcp pool router17 network 192.168.0.0 255.255.0.0 default-router 192.168.0.1 ! ! ! ip cef no ipv6 cef ! ! ! ! ! ! ! ! ! ! ! ! ! ! ! ! ! ! interface Loopback1 ip address 192.168.10.1 255.255.255.0 ! interface FastEthernet0/0 ip address 47.0.0.25 255.255.255.252 duplex auto speed auto ! interface FastEthernet1/0 no ip address duplex auto speed auto shutdown ! interface Serial2/0 no ip address clock rate 2000000 shutdown ! interface Serial3/0 no ip address clock rate 2000000 shutdown ! interface FastEthernet4/0 no ip address shutdown ! interface FastEthernet5/0 no ip address shutdown ! router ospf 1 log-adjacency-changes network 47.0.0.24 0.0.0.3 area 47 ! ip classless ! ip flow-export version 9 ! ! ! ! ! ! ! ! line con 0 ! line aux 0 ! line vty 0 4 login ! ! ! end Router17#

# Router 16

Router16#sh run Building configuration... Current configuration : 1052 bytes ! version 12.2 no service timestamps log datetime msec no service timestamps debug datetime msec no service password-encryption ! hostname Router16 ! ! ! ! ! ip dhcp pool router16 network 192.168.66.0 255.255.255.0 default-router 192.168.66.1 ip dhcp pool router16-2 network 192.168.48.0 255.255.240.0 default-router 192.168.16.1 ! ! ! ip cef no ipv6 cef ! ! ! ! ! ! ! ! ! ! ! ! ! ! ! ! ! ! interface Loopback2 ip address 192.168.11.1 255.255.255.0 ! interface FastEthernet0/0 ip address 48.0.0.26 255.255.255.252 duplex auto speed auto ! interface FastEthernet1/0 no ip address duplex auto speed auto shutdown ! interface Serial2/0 no ip address clock rate 2000000 shutdown ! interface Serial3/0 no ip address clock rate 2000000 shutdown ! interface FastEthernet4/0 no ip address shutdown ! interface FastEthernet5/0 no ip address shutdown ! router ospf 1 log-adjacency-changes network 48.0.0.24 0.0.0.3 area 48 ! ip classless ! ip flow-export version 9 ! ! ! ! ! ! ! ! line con 0 ! line aux 0 ! line vty 0 4 login ! ! ! end Router16#

# Router 15

Router15#sh run Building configuration... Current configuration : 815 bytes ! version 12.2 no service timestamps log datetime msec no service timestamps debug datetime msec no service password-encryption ! hostname Router15 ! ! ! ! ! ! ! ! ip cef no ipv6 cef ! ! ! ! ! ! ! ! ! ! ! ! ! ! ! ! ! ! interface FastEthernet0/0 ip address 49.0.0.29 255.255.255.252 duplex auto speed auto ! interface FastEthernet1/0 no ip address duplex auto speed auto shutdown ! interface Serial2/0 no ip address clock rate 2000000 shutdown ! interface Serial3/0 no ip address clock rate 2000000 shutdown ! interface FastEthernet4/0 no ip address shutdown ! interface FastEthernet5/0 no ip address shutdown ! router ospf 1 log-adjacency-changes network 49.0.0.28 0.0.0.3 area 49 ! ip classless ! ip flow-export version 9 ! ! ! ! ! ! ! ! line con 0 ! line aux 0 ! line vty 0 4 login ! ! ! end Router15#

# Router 14

Router14#sh run Router14#sh running-config Building configuration... Current configuration : 1052 bytes ! version 12.2 no service timestamps log datetime msec no service timestamps debug datetime msec no service password-encryption ! hostname Router14 ! ! ! ! ! ip dhcp pool router14 network 192.168.32.0 255.255.224.0 default-router 192.168.32.1 ip dhcp pool router14-2 network 192.168.31.0 255.255.255.0 default-router 192.168.31.1 ! ! ! ip cef no ipv6 cef ! ! ! ! ! ! ! ! ! ! ! ! ! ! ! ! ! ! interface Loopback3 ip address 192.168.12.1 255.255.255.0 ! interface FastEthernet0/0 ip address 49.0.0.26 255.255.255.252 duplex auto speed auto ! interface FastEthernet1/0 no ip address duplex auto speed auto shutdown ! interface Serial2/0 no ip address clock rate 2000000 shutdown ! interface Serial3/0 no ip address clock rate 2000000 shutdown ! interface FastEthernet4/0 no ip address shutdown ! interface FastEthernet5/0 no ip address shutdown ! router ospf 1 log-adjacency-changes network 49.0.0.24 0.0.0.3 area 49 ! ip classless ! ip flow-export version 9 ! ! ! ! ! ! ! ! line con 0 ! line aux 0 ! line vty 0 4 login ! ! ! end Router14#